OVT has its own extension ‘.ovt’.

The instructions in OVT are divided into four categories:

- 00xx xxxx: immediate (imme)

- 01xx xyyy: copy

- 10--  -xxx: operations (ops)

- 11--  -xxx: jump

A dash means that that bit doesn’t have any effect on the instruction, and ‘x’ and ‘y’ are numbers that control what the instruction does.

Instructions:

| keyword | description | instruction |
| --- | --- | --- |
| Imme <x> | Load a 6bit number into reg0 | 00xx xxxx |
| Copy <x> <y> | Copy reg(x) to reg(y) | 01xx xyyy |
| Or | Or reg1 and reg2, store the result in reg3 | 10-- -000 |
| Nand | Nand reg1 and reg2, store the result in reg3 | 10-- -001 |
| Nor | Nor reg1 and reg2, store the result in reg3 | 10-- -010 |
| And | And reg1 and reg2, store the result in reg3 | 10-- -011 |
| Add | Add reg1 and reg2, store the result in reg3 | 10-- 100 |
| Sub | Sub reg1 and reg2, store the result in reg3 | 10-- -101 |
| Jump false | Don’t jump. | 1100 0000 |
| Jump >0 | If reg3 >0, jump to reg0 | 1100 0001 |
| Jump =0 | If reg3 =0, jump to reg0 | 1100 0010 |
| Jump >=0 | If reg3 >=0, jump to reg0 | 1100 0011 |
| Jump <0 | If reg3 <0, jump to reg0 | 1100 0100 |
| Jump !=0 | If reg3 ≠0, jump to reg0 | 1100 0101 |
| Jump <=0 | If reg3 <=0, jump to reg0 | 1100 0110 |
| Jump | Jump to reg0 | 1100 0111 |

OVT isn’t case sensitive so both ‘Nand’ and ‘NAND’ work.

Registers:

| Name | Function |
| --- | --- |
| Reg0 | Immediate |
| Reg1 | First operand |
| Reg2 | Second operand |
| Reg3 | Result of operation |
| Reg4 | - |
| Reg5 | - |
| Reg6 | PROGRAM COUNTER, this controls which instruction is executed. |
| Reg7 | Not really a register. This is an input from outside the computer when used as source and an output when used as destination. |

Const <name> <value>: Every .ovt program can start with a few lines that start with ‘const’.

assigns the value <value> to <name>. From then on you use <name> instead of <value>.

Label <name>: assigns the line number of the next instruction to <name>. This is useful for loops.

; (comments): everything after ‘;’ is seen as a comment (until a new line).

Example of code with and without consts, labels and comments:

| Code without | Code with |
| --- | --- |
| copy 7 3 imme 0 jump =0 copy 3 7 | const input 7 ;input as name for reg7 const output 7 ;output " " " " label loop ;loop until input !=0 copy input 3 imme loop jump =0 copy 3 output ;output the first non-zero input |

This program copies an input (reg7) to reg3 and uses the immediate value ‘loop’ (0) to loop. It loops until the input isn’t 0 and then outputs that input.

The second version of the code looks worse for small snippets like this but for larger programs it’s a vast improvement. (Example: 14)

The above code is compiled to the following machine code:

- Binary: 01111011 00000000 11000010 01011111

- Hex: 7b 00 c2 5

All numbers in the code are hexadecimal. In ‘Imme 11’, ‘11’ = 0x11 = 17